# Myster Discord-Like Voice Chat
Design Exploration

Working design note • 4 October 2026

Audience: Myster maintainer and implementation/design AI

Status: exploratory design with several decisions effectively locked in. The central Myster-server relay architecture, Myster-based signaling, Swing UI, and use of WebRTC for the media engine are the current direction. The main technical risk is custom transport integration with Myster's existing UDP/TCP multiplexing.

# 1. Purpose and Scope

This document captures the current design exploration for adding Discord-like voice channels to Myster. It is intended to preserve the reasoning, current architectural decisions, useful library/API knowledge, and unresolved implementation questions so that future work can continue without reconstructing the discussion.

The immediate goal is voice chat, not video. The design deliberately keeps Myster responsible for identity, communities/channels, permissions, server selection, signaling and user experience, while delegating real-time audio transport and processing to an existing WebRTC implementation.

# 2. Decisions and Direction So Far

Myster server is the central media relay point for a voice channel. Clients do not need to form an all-to-all peer mesh.

Myster's existing chat/control facilities should carry voice-channel discovery, join/leave state and WebRTC signaling messages.

Use WebRTC as the media engine rather than implementing Opus transport, jitter buffering, loss recovery, echo cancellation and related VoIP machinery independently.

The leading Java library is dev.onvoid/webrtc-java, a JNI wrapper around the native WebRTC C++ implementation.

The Myster desktop UI remains Swing/AWT. WebRTC does not require JavaFX.

WebRTC audio capture/playback devices should be enumerated and selected through the WebRTC library, rather than trying to map JavaSound mixers to WebRTC devices.

Myster's existing transport multiplexing should remain in control: UDP packets need Myster's 16-bit protocol discriminator; TCP uses Myster's existing 32-bit protocol selection handshake once at connection establishment.

The critical proof-of-concept is whether WebRTC's native network/socket layer can be adapted cleanly enough to use Myster's UDP and TCP transport conventions.

# 3. Target Architecture

A voice channel is a Myster-level concept. A client learns about and joins it through the ordinary Myster server/control path. WebRTC is then used as the media session between that client and the Myster server.

MYSTER COMMUNITY / CHAT SERVER
                               |
             +-----------------+-----------------+
             |                 |                 |
          Client A          Client B          Client C
             |                 |                 |
             +---- WebRTC -----+---- WebRTC -----+
                   media sessions to server

Control plane: Myster chat/protocol messages
Media plane:   WebRTC audio sessions through Myster transport integration

The server behaves conceptually like an SFU/relay: each client sends an encoded audio stream to the server, and the server forwards the appropriate participant streams to other members of the voice channel. The design should avoid decode/re-encode or server-side audio mixing unless a later feature requires it.

# 4. Signaling and Voice-Channel Control

WebRTC intentionally does not define the application signaling protocol. Myster should use its own authenticated protocol/channel messages for this. The exact message schema is not yet fixed.

Advertise that a community/channel supports voice.

Join and leave a voice channel.

Notify other members about presence/state changes.

Carry SDP offer/answer information.

Carry ICE candidate information if ICE is retained in the chosen integration.

Associate the WebRTC session with the Myster server identity, channel identity and current authenticated user/server session.

Carry mute/deafen/presence metadata that is application state rather than media transport state.

Myster client                      Myster server
     |                                   |
     |--- JoinVoice(channel) ----------->|
     |<-- VoiceJoinAccepted / config ----|
     |--- WebRTC offer ----------------->|
     |<-- WebRTC answer -----------------|
     |<-> ICE/candidate data as needed <-|
     |                                   |
     |====== WebRTC media established ===|

# 5. Java/WebRTC Library Choice

## 5.1 Leading Candidate: webrtc-java

The current candidate is dev.onvoid/webrtc-java. It is a Java wrapper over the native WebRTC API, using a thin JNI layer. It provides desktop support for Windows, macOS and Linux and ships platform-specific native libraries through Maven artifacts. The project dates back to 2019 and remains actively maintained.

This means Myster would no longer be purely Java at runtime for voice-enabled builds; native binaries become a packaging concern. That is acceptable in principle, but should be treated explicitly in packaging, Flatpak/jpackage work, architecture support and release testing.

## 5.2 No JavaFX Requirement

The WebRTC library is not a UI toolkit and does not require Myster to adopt JavaFX. Myster can keep its existing Swing/AWT UI. Any JavaFX code associated with examples or demos is optional application UI, not a requirement of the core media engine.

## 5.3 What the Library Provides

PeerConnection API and WebRTC session negotiation.

Audio capture and playout through native platform audio APIs.

Opus-capable real-time audio transport as part of WebRTC.

Echo cancellation, noise suppression, automatic gain control and other voice-processing components.

Jitter buffering, packet-loss handling and real-time transport behavior implemented by WebRTC.

Audio-device enumeration and selection.

Headless audio support useful for a server that must not touch physical audio hardware.

Connection statistics useful for future diagnostics (packet loss, jitter, RTT, bitrate, etc.).

# 6. Java Usage Model

At the Java layer, a client creates a PeerConnectionFactory, configures audio, creates an RTCPeerConnection, adds an audio track, and forwards SDP/ICE signaling data through Myster. The following is deliberately schematic: it documents the shape of the API rather than being a copy/paste implementation.

AudioDeviceModule adm = new AudioDeviceModule();
adm.setRecordingDevice(selectedMicrophone);
adm.setPlayoutDevice(selectedSpeaker);

PeerConnectionFactory factory = new PeerConnectionFactory(adm);

AudioOptions audioOptions = new AudioOptions();
audioOptions.echoCancellation = true;
audioOptions.noiseSuppression = true;
audioOptions.autoGainControl = true;

AudioTrackSource source = factory.createAudioSource(audioOptions);
AudioTrack microphone = factory.createAudioTrack("myster-voice", source);

RTCPeerConnection peer = factory.createPeerConnection(configuration, observer);
peer.addTrack(microphone, List.of("myster-voice"));

// createOffer()/createAnswer() output is sent using Myster signaling.
// onIceCandidate() output is likewise carried by Myster if ICE is used.

The Myster server would create corresponding peer connections without using a physical microphone or speaker. webrtc-java now documents a HeadlessAudioDeviceModule specifically for server-side/headless operation. The exact best server construction for an SFU-style forwarding implementation still needs a prototype.

# 7. Audio Processing

One of the strongest reasons to use WebRTC is that voice-processing functionality is already built for this use case. webrtc-java exposes audio processing including echo cancellation, noise suppression, gain control and high-pass filtering. Echo cancellation is particularly valuable because it uses knowledge of playout audio as the reference signal when removing speaker leakage from microphone capture.

Initial Myster defaults should likely enable echo cancellation, noise suppression and automatic gain control. Advanced settings can expose overrides later if users have unusual hardware or professional audio setups.

# 8. Swing UI Requirements

WebRTC supplies no Myster user interface. Myster owns all UI. A first implementation needs two UI surfaces: voice-channel controls and an audio-settings panel.

## 8.1 Voice Channel Controls

Join / leave voice channel

Mute / unmute microphone

Deafen / undeafen

Participant list

Speaking/activity indication

Connection/error state

## 8.2 Audio Device Settings

Input-device selector (microphone)

Output-device selector (speaker/headphones)

Input level meter

Microphone test / optional loopback test

Refresh/update on device hot-plug

Optional advanced controls for echo cancellation, noise suppression and AGC

Current webrtc-java documentation exposes device enumeration through MediaDevices and selection through AudioDeviceModule. Device queries should be performed off the Swing EDT. The library also exposes device-change notification, which can be used to refresh selectors when USB/Bluetooth devices appear or disappear.

List<AudioDevice> microphones = MediaDevices.getAudioCaptureDevices();
List<AudioDevice> speakers    = MediaDevices.getAudioRenderDevices();

AudioDeviceModule adm = new AudioDeviceModule();
adm.setRecordingDevice(selectedMicrophone);
adm.setPlayoutDevice(selectedSpeaker);

# 9. Integration With Myster's Existing Transport Multiplexing

This is the critical integration area. Myster already multiplexes multiple application protocols over common UDP/TCP infrastructure. Voice should fit that model rather than opening an unrelated second networking architecture if practical.

## 9.1 UDP

Myster's UDP stack requires a 16-bit protocol discriminator at the start of every UDP datagram so a shared UDP socket can demultiplex packets into the correct protocol handler. Therefore every WebRTC datagram sent through Myster must be transformed as follows:

On the wire:

+----------------------+-----------------------------------+
| Myster protocol ID   | Original WebRTC UDP datagram      |
| unsigned 16-bit      | STUN / DTLS / SRTP / etc.         |
+----------------------+-----------------------------------+
          2 bytes                   N bytes

Transmit path: WebRTC emits a datagram → Myster transport prepends the 16-bit WebRTC/voice protocol ID → shared Myster UDP socket sends it.

Receive path: shared Myster UDP socket receives a datagram → Myster dispatcher reads/removes the 16-bit protocol ID → remaining datagram is delivered unchanged to the WebRTC network layer.

The two-byte overhead is negligible for voice traffic. The important requirement is preservation of datagram boundaries and source/destination addressing information expected by WebRTC.

## 9.2 TCP

TCP is simpler at the Myster protocol level because Myster selects the protocol once per TCP connection. The existing convention is a 32-bit protocol selector from client to server followed by a one-byte server response. After successful selection, that TCP connection belongs to the selected protocol and can carry that protocol's byte stream.

TCP connection establishment

client -> server : int32 MYSTER_WEBRTC_PROTOCOL
server -> client : byte  ACCEPT/STATUS

----------------------------------------------
From this point onward the connection is owned
by the WebRTC transport; no Myster prefix is
required for each media packet/frame.
----------------------------------------------

Therefore Myster should not add a per-packet prefix to the TCP media stream. WebRTC (or its ICE/TCP transport layer) must retain responsibility for whatever stream framing it expects internally. The Myster handshake happens only before WebRTC is allowed to treat the connection as established.

## 9.3 Required Custom Networking Hook

The desired implementation is to make WebRTC believe it is using ordinary packet sockets while a Myster-aware socket implementation performs the protocol adaptation underneath it.

WebRTC native stack
                               |
                     custom packet/socket layer
                               |
              +----------------+----------------+
              |                                 |
             UDP                               TCP
   prepend/remove uint16             connect normally, then
   Myster protocol ID                perform Myster int32 +
   on every datagram                 byte handshake once
              |                                 |
              +----------------+----------------+
                               |
                      Myster network stack

Upstream WebRTC has packet/socket factory abstractions and configurable ICE networking. However, the current public webrtc-java documentation does not expose an obvious Java-level custom PacketSocketFactory/AsyncPacketSocket hook. This must be verified against the exact version chosen for implementation. A small addition to the library's JNI layer may be required.

# 10. Encryption and Identity

WebRTC already encrypts its media transport (DTLS/SRTP in the normal WebRTC model). Myster should avoid wrapping media in a second encryption layer unless the transport design later demonstrates a concrete need for it. Double encryption would add complexity while providing little benefit if identity binding is done correctly.

The important Myster-specific requirement is binding the WebRTC session to the authenticated Myster server/user/channel context. Signaling can carry fingerprints/nonces/session identifiers through an already-authenticated Myster connection so the client knows that the media endpoint belongs to the Myster server it intended to join.

Exact cryptographic binding is not yet designed and should be treated separately from the basic media transport proof-of-concept.

# 11. Server Relay / SFU Direction

The architecture is now assumed to be Myster-server-centric rather than peer mesh. The server should preferably forward encoded participant streams rather than mix them. This keeps CPU cost lower, avoids quality loss from decode/re-encode, and allows each client to control per-user volume locally.

A future implementation must determine whether webrtc-java exposes the encoded-media forwarding primitives needed for an efficient SFU directly, or whether the server should use lower-level RTP/encoded-frame hooks. This is a separate question from the socket integration, but it is the next major server-side feasibility check after transport.

# 12. UDP/TCP Behavior and Fallback

Myster already has reason to care about networks where UDP is blocked. The voice design should therefore preserve a TCP-capable path. The intended behavior is:

Prefer UDP media through the shared Myster UDP transport, using the 16-bit per-datagram protocol discriminator.

If UDP cannot be established, allow a TCP path where the client opens a Myster TCP connection, performs the 32-bit protocol selection / one-byte response handshake, and then hands the resulting byte stream to WebRTC.

Do not invent a Myster per-packet framing layer over the established TCP stream unless WebRTC integration forces one; let WebRTC own its stream framing.

The exact way ICE candidate gathering and transport fallback are constrained to these Myster-aware sockets remains part of the prototype work.

# 13. Known Risks and Open Questions

| Question / Risk | Current Understanding | Required Action |
| --- | --- | --- |
| Custom socket injection | Upstream WebRTC supports networking abstractions, but a suitable Java-level hook is not obvious in webrtc-java. | Prototype custom UDP/TCP socket integration; extend JNI wrapper if required. |
| UDP shared socket semantics | Myster needs a uint16 prefix on every datagram and must preserve source/destination metadata. | Verify ICE/WebRTC accepts the shared-socket/demux model and can receive injected datagrams correctly. |
| TCP handoff | Myster can select the protocol once, then give the stream to WebRTC. | Verify custom WebRTC TCP socket can perform the Myster handshake before reporting connected. |
| Server forwarding / SFU | Central relay is locked in conceptually; forwarding without mixing is preferred. | Confirm encoded RTP/frame forwarding hooks and multi-party topology in the chosen WebRTC API. |
| ICE/STUN/TURN scope | Central server topology may allow a greatly simplified candidate model. | Prototype before deciding how much standard ICE/STUN/TURN infrastructure remains necessary. |
| Identity binding | Myster authentication/signaling can bind the media session to server/channel identity. | Design exact fingerprint/session-nonce/signature scheme after transport works. |
| Native packaging | webrtc-java ships native binaries; Myster voice builds become platform-specific at runtime. | Test Windows/Linux/macOS packaging and Flatpak/jpackage integration. |
| Audio-device lifecycle | Library exposes enumeration, selection and hotplug events. | Define preference persistence and behavior when selected hardware disappears. |

# 14. Recommended Proof-of-Concept Order

Create a minimal two-endpoint webrtc-java voice connection using ordinary networking. Confirm microphone capture, playout, echo cancellation and device selection.

Run a headless server-side endpoint and confirm it can receive/send audio without touching physical audio devices.

Implement or expose the custom native packet/socket layer. First prove UDP with the Myster 16-bit prefix and shared socket demultiplexer.

Prove TCP fallback using the existing Myster int32 protocol selector + one-byte response before WebRTC takes over the stream.

Only after transport works, prototype the server relay/SFU behavior with two or three clients.

Integrate SDP/ICE exchange into Myster chat/control messages.

Add the Swing voice-channel UI and audio settings panel.

Add identity/session binding, diagnostics, failure handling and packaging.

# 15. Explicit Non-Goals for the First Voice Implementation

Video calling or screen sharing.

Server-side audio mixing.

Replacing Myster chat/signaling with WebRTC data channels.

Building a new audio codec or jitter buffer.

Building custom acoustic echo cancellation.

Reworking Myster into JavaFX.

Designing a generic TURN service before proving it is actually needed for the central-server topology.

# 16. Sources Checked During This Exploration

These are implementation references rather than normative Myster requirements:

webrtc-java project and repository: https://github.com/devopvoid/webrtc-java

webrtc-java documentation: https://jrtc.dev/

Getting Started / media tracks / AudioOptions: https://jrtc.dev/guide/get-started

Audio device selection: https://jrtc.dev/guide/audio/audio-devices

Media device enumeration and hotplug events: https://jrtc.dev/guide/media/media-devices

Audio processing: https://jrtc.dev/guide/audio/audio-processing

Headless audio: https://jrtc.dev/guide/audio/headless-audio

ICE / port allocator configuration: https://jrtc.dev/guide/networking/port-allocator-config

# 17. Current Bottom Line

The design is promising and fits Myster well. WebRTC can supply the difficult real-time audio machinery while Myster keeps ownership of identities, communities, channels, signaling and UI. The central-server relay model also matches Myster's architecture better than an N-way peer mesh.

The project should not proceed to substantial UI or channel-protocol implementation until one technical spike succeeds: make WebRTC media traverse Myster's existing transport conventions — uint16 protocol prefix per UDP datagram and the one-time int32/byte TCP protocol handshake. If that works cleanly, the remaining work becomes conventional application integration rather than a fundamental architecture problem.